2.1 Types and methods of data transmission ANSWER KEY

LAYER 1: THEORY

•  A small unit into which data is divided for transmission is a packet.

•  The section of a data packet containing information about its contents and destination is the packet header.

•  The actual data being carried in a data packet is the payload.

•  The section of a data packet containing information about error checking is the trailer.

•  The unique addresses assigned to devices when they connect to a network are called IP addresses.

•  The method in which data packets can take individual pathways across a network is called packet switching.

•  Serial transmission sends data one bit at a time down a single wire.

•  The method that sends multiple bits at a time down multiple wires is called parallel transmission.

•  The transmission method that sends data in one direction only is called simplex.

•  The transmission method that sends data in both directions, but only one direction at a time, is called half-duplex.

•  The transmission method that sends data in both directions at the same time is called full-duplex.

•  The industry standard Universal Serial Bus, used to transmit data, is abbreviated to USB.

•  The collective name for using a USB cable plugged into a USB port to transfer data between devices is a USB connection.

LAYER 3: COURSEBOOK PRACTICE

1  Identify two items of data that would be included in the header of a packet of data.  [2]

Any two from:

• destination address

• packet number

• originator’s address.

2  Explain why and how packets of data are reordered after packet switching has been used to transmit data across a network.  [2]

Any two from:

• Each data packet is sent individually, possibly along a different route, so it could arrive out of sequence.

• The receiving device reads the packet number from the header and reorders the packets based on this value.

3  Describe how data is transmitted using serial half-duplex data transmission.  [4]

Award 1 mark for each point.

• Bits of data are sent down a single wire.

• Bits of data are sent one at a time.

• Data can be sent in both directions

• … but only one direction at a time.

4  Give two benefits of using the USB interface to connect hardware devices to a computer.  [2]

Any two from:

• The cable can only be inserted one way into the port, so this helps prevent incorrect connections.

• It supports different transmission speeds.

• It can power another device whilst transmitting data.

• Devices are automatically detected when connected.

Explain each of the following computer terms:

5(i)  packet switching  [2]

A message is divided into packets. These travel independently and may follow different routes through the network.

5(iv)  universal serial bus  [2]

USB is an industry standard for connecting computers and devices through USB ports using serial data transfer.

A file server is used as a central data store for a network of computers.
Rory sends data from his computer to a file server that is approximately 100 metres away. It is important that the data is transmitted accurately. Rory needs to be able to read data from and write data to the file server at the same time.

6(a)(i)  Use ticks (✓) to identify the most suitable data transmission methods for this application.  [2]

| Method 1 | Tick ✓ | Method 2 | Tick ✓ |
| --- | --- | --- | --- |
| Serial | ✓ | Simplex |  |
| Parallel |  | Half-duplex |  |
|  |  | Duplex | ✓ |

6(a)(ii)  Explain why your answer to a i is the most suitable data transmission.  [4]

• Serial is reliable across long distances.

• Its bits remain synchronised.

• Full-duplex permits simultaneous transfer in both directions.

• Simplex and half-duplex cannot support reading and writing at the same time.

Maisey purchases a new router and attaches it to her computer. The connection she sets up uses duplex data transmission.

7(a)  Five statements are given about duplex data transmission.
Tick (✓) to show if the statement is True or False.  [5]

| Statement | True ✓ | False ✓ |
| --- | --- | --- |
| Duplex data transmission can be either serial or parallel | ✓ |  |
| Duplex data transmission is when data is transmitted both ways, but only one way at a time |  | ✓ |
| Duplex transmission is always used to connect a device to a computer |  | ✓ |
| Duplex data transmission is when data is transmitted both ways at the same time | ✓ |  |
| Duplex data transmission automatically detects any errors in data |  | ✓ |

Maisey’s computer uses an integrated circuit (IC) for data transmission that sends multiple bits at the same time.

7(b)  State whether the IC uses serial or parallel data transmission.  [1]

Parallel.

Maisey purchases a new printer and connects it to her computer using the USB port.

7(c)  Explain two benefits of using a USB connection.  [4]

• A connector that fits in one direction reduces the risk of an incorrect connection.

• USB supplies power through its cable, allowing a device to operate without a separate power source.